ADA-NET Internet Setup

For Windows 95;

1.	Start Windows 95.  Then,

2.	From My Computer/Control Panel/System/Device make sure that there is no device conflict.  (If there is a conflict you would see "?" or "!" right next to device.)

If there is a conflict, take your computer to where you purchased it or if you are a professional user try to solve the conflict problem by choosing the right IRQ and adressess.

- My Computer/Contro Panel/Modern;

Is there any modem listed ? If there is no modem listed add one by clicking on the "Add" button (if your modem is an external modem turn it on before you add).

System will detect your modem.  If the system can't find the modem then there is a problem with your modem.  In such a problem contact your system provider.  If there is a modem on the list go in to "Diagnostics" tabset and by choosing the port your modem is connected to click on the "More Info" button.

If you see a window that shows an information about your modem that means that the modem setup is completed.  If you get an error then there is a problem with your modem.

- My Computer/Control Panel/Network;

On this window do you see "Dial-Up Adapter" and "TCP/IP" listed ?

If not, choose Dial-Up Adapter from Add/Adapter/Microsoft.

Then choose TCP/IP form Add/Protocol/Microsoft.  Click on OK.  By answering yes to "Do you want your computer restarted?"  restart your system.

- If there is an "Internet" icon in My Computer/Control Panel, then Internet/Connection;

In this section mark the "Connecto to the Internet as needed" checkbox (if not marked).

Click on the "Add" button.

Type "AdaNET" in the window that comes up, click on the "Next" button, in this window type 822 in "Area Code" and  3144002 in "Telephone Number".

Click on the "Next" button twice and your "AdaNET" connection is completed.

Click on the "Properties" button.  Click on the "Configure" button in the window that comes up and go in to "Option" tabset.

Mark the "Display modem status" checkbox and unmark "Bring up terminal window before dialing" and "Brign up terminal window after dialing".

Click on the "OK" button.  Click on the "Server Type" button.  In this window just mark the "Enable Software Compression" and "TCP/IP" checkboxes and leave the rest unmarked.

Go in to "TCP/IP Settings" section.

In this window "Server Assigned IP Adress" should be chosen, on the lower section of the window "Specify Name Server Addresses" radio button should be chosen;

For the Primary DNS you should enter : 213.232.0.18

For the Secondary DNS you should enter : 195.112.155.130

On the bottom of the same window "Use IP Header Compression" and "Use Default Gateway On Remote Network" should be marked too.

- My Computer/Dial-Up Networking/AdaNET/Dial-Properties;

In this window you should enter the area code of the city you live in "Area Code" and Turkey 90 in "I am in".

To acces an outside line enter the number to access to an outside line followed by a comma in the appropriate box.  Comma is to wait for one second after getting the outside line access.

If you made it upto here your system is ready to connecto to internet.

Restart you system.

Go into My Computer/Dial-Up Networking/AdaNET.  Click on the "Connect" button to connect to AdaNET.  Once your computer is connected to our system you are ready to user you internet programs such Netscape, Internet Explorer and vs..  If you don't have any kind of internet programs you are going to have to install them.  Also before clicking on the "Connect" button make sure that the username and the password that was given to you by AdaNET is typed in to appropriate boxes.